CPD Questions Winter 2015

Paper 1: Three-dimensional carotid ultrasound: Reproducibility in plaque and artery volume measurements. Hayrapet Kalashayan et at.

Question 1

What errors are associated with 2D measurements of assessment of a stenosis:

Answer: Doppler measurements are susceptible to a variety of technical and hemodynamic factors meaning the technique is operator-dependent and may vary further between different machines and manufacturers.

2 What is the maximum number of stacked consecutive transverse slices that can be used?

Answer: Depending on the plaque length, the number of slices can be increased to a maximum of 15 by altering the inter-slice distance.

3 What prevented measurement of plaque and artery volume in 31 cases?

Answer: Accurate assessment was impossible in the remaining 31 cases, mainly due to the presence of heavy calcification and acoustic shadowing. In a small number of cases this was due to hypo-echogenic plaques such that the inner lumen could not be visualised accurately on B mode.

Paper 2: Imaging of plaque perfusion using contrast-enhanced ultrasound-Clinical significance. Edoardo Vinenzini et al:

Question 5

What types of conventional radiological imaging have been used to assess plaque metabolic activity?

Answer: ..conventional radiological imaging, such as Computerised Tomography Angiography, Magnetic Resonance Angiography and also 18-FDG Positron Emission Tomography have focused on the evaluation of the “plaque metabolic activity”…

6 When was a plaque considered ulcerated?

Answer: Plaque was considered ulcerated when and excavation of ≥ 2mm was observed.

7 When was vascularisation of plaques not detected?

Answer: Vascularisation was not detected in the hyperechoic with acoustic shadow calcific tissue, nor in the hypoechoic necrotic and hemorrhagic areas.

8 What is one limit to this technique of assessing plaque vascularization?

Answer: One limit of this approach of this technique regards the modality for the evaluation of the vascularisation: at present, a method of a real numerical objective quantification of the global “plaque perfusion” is indeed not available for carotid plaques.