A carotid ultrasound reveals very poor images of the lumen due to calcification of the atheroma. The reason for this can be best explained by the following statement.

- the calcium molecules cause scattering of the ultrasound beam

- the calcium molecules absorb all of the ultrasound energy

- the calcium causes the beam to be refracted away from the transducer

- there is a large discrepancy between  the acoustic  impedance of  the calcified

atheroma and that of the surrounding tissue

- the calcium molecules resonate at a much higher frequency than the incident frequency. The reflected frequency is therefore too high to be picked up and displayed as an image