Lower limb arterial duplex assessment

Aim: To identify the sites and severity of arterial disease in the lower limb.

Procedure: Obtain a signed request from a clinician to perform the examination.

# Identify the patient by at least 2 points of reference (i.e full name, d.o.b., address) and explain the procedure to be conducted. Ensure the patent is suitably dressed and positioned to proceed with the examination, positioning is usually supine but other positions may be require depending on circumstances.

Note any previous investigations and be aware of the segment of the lower limb in which there is particular interest. If ABPI’s have not been reported previously, these should be performed at start of test to confirm that a duplex scan is appropriate. ( See protocol for conventional ABPI’s if suitable).

Using the appropriate scanner, transducer and settings, obtain spectral waveforms from the anterior and posterior tibial arteries just above the level of the malleolus. Confirm that flow patterns on spectral trace correspond to the reported ABPI’s.  If conventional ABPI’s are not suitable ie: ankle ulceration, leg swelling and densely calcified tibial vessels, then descriptions of ankle waveform patterns should be noted (eg: damped/sharp).

If ABPI’s and distal flow patterns are normal, further imaging is not necessary unless specific information such as, present of aneurysmal disease is suspected or if a Doppler treadmill test has been requested which subsequently demonstrates a significant post exercise drop in ABPI.

If ABPI’s and distal flow patterns are damped or abnormal, each of these vessels should be followed up to origin with use of colour Doppler to highlight the presence and location of any disease within the vessel. Images should be recorded of lesions believed to be significant. Obtain a spectral Doppler sonogram of the pattern of flow through any stenosed segment. Estimate the degree of stenosis noted using the velocity ratio grading criteria outlined in table (A) in the appendix section.

After appropriate repositioning, the popliteal artery should be imaged and a sonogram obtained. As for the tibial segments, note any significant disease, applying velocity grading criteria. Measurements of the diameter of the popliteal artery should be recorded if aneurysmal disease is suspected. The popliteal artery should be followed proximally to the above knee level and distally through into the tibio-peroneal trunk if possible.

# The imaging should now be directed to the femoral segment. Image the common femoral artery (CFA) in longitudinal view. The CFA should be imaged and a sonogram obtained. As for the previous segments, note any significant disease. Additionally, if flow patterns are damped within the proximal CFA or if symptoms suggest proximal disease, this should be noted and investigation of the aorto-iliac segment to follow.

Move down to the CFA to the femoral bifurcation and image the origins of the superficial femoral artery (SFA) and profunda femoris artery. Note any segments of disease and/or velocity changes. The profunda femoris need not be followed beyond origin unless specifically requested.

In longitudinal view using colour Doppler follow the SFA and note any lesions or occlusions as for CFA, obtain spectrograms of the flow pattern within the SFA, recording the patterns if significant changes are observed. Follow the SFA to the adductor hiatus as distally within the thigh as possible, noting any segments where imaging was not possible.

If disease of the aorto-iliac segment is indicated or specifically requested, imaging of this segment with the appropriate transducer and patient positioning should be conducted.

Image the abdominal aorta from as proximal as can be clearly seen in the transverse and longitudinal axis noting the diameter of the vessel and confirm patency of the vessel using colour and spectral Doppler imaging. Again note any segments of disease and/or velocity changes using the aforementioned grading criteria reference. The abdominal aorta should be followed distally to the aorto-iliac bifurcation and both iliac segments imaged accordingly in turn from origin of the CIA through to the origin of the CFA, inclusive of the EIA. Again noting any segments of disease and/or velocity changes.

Explain the findings of the examination in a manor appropriate to the individual patient.

Images should be reviewed and checked before ending the exam and then sent to PACS. As soon as it is practically possible, write up the findings of the investigations on CRIS. If specifically requested a hard copy of the report should be sent to the referring physician. When potentially limb threatening or unexpected findings are made, the clinical team should be contacted directly.

Reference

Peripheral Vascular Ultrasound How When and Why. Thrush and Hartshorne. Chapter 9.

Guidelines of Working Practice for the Vascular Scientist.

APPENDIX

Table (A)

Grading lower limb arterial disease using velocity ratio criteria

Diameter	Velocity ratio 		Comments

reduction

0-49%		<2			Waveform is triphasic, but mild spectral broadening and

an increase in end diastolic velocities are recorded as the

degree of narrowing approaches

50-74%        	>2			Waveforms tend to become bi- or monophasic.  There is

an increase in end diastolic velocity.  Spectral broadening

is present.  Flow disturbance and some damping is

recorded distal to the stenosis

75-99%        	>3			Waveform is usually monophasic with a significant

increase in end diastolic velocity.  Marked turbulence and

spectral broadening are demonstrated.  Flow is damped

distal to the stenosis

Occluded      	No flow			Doppler waveforms proximal to an occlusion often

Detected			demonstrate a high resistance flow pattern