Protocol Superficial venous imaging

Lower limb superficial venous assessment - incompetence

Aim: To clarify the anatomy of the superficial venous system and identify the site and severity of reflux.

Procedure:

- Obtain a signed request from a clinician to perform examination

# Identify the patient by at least 2 points of reference (i.e. full name, d.o.b , address) and explain the procedure.

# Ensure the patent is suitably dressed and positioned to proceed with the examination, positioning is usually standing on step or platform but other positions may be required depending on circumstances.

# Note any previous investigations, relevant history or previous surgery or DVT and be aware of the segment of the lower limb in which there is particular interest.

SSV System.

Using the appropriate scanner, transducer and setting, and a good quality ultrasound gel, image the popliteal vein in longitudinal view. Check the deep vein below the sapheno-popliteal junction (SPJ) for incompetence using colour Doppler during distal calf compression and release. Spectral Doppler should be used to record the severity of any reflux or in cases where borderline reflux is suspected.

Move to a transverse view of the popliteal fossa and note the presence (dependant on previous surgery) of the short saphenous vein (SSV). Check the SSV and main gastrocnemius veins for reflux using calf compression with Doppler assessment.

If any SSV reflux is noted, identify and mark the level of the SPJ with a marker pen. The SSV should be imaged distally to Achilles tendon level to exclude any further sources of incompetence distally such as a communicating branch of the LSV system or gastrocnemius vein perforators. The SSV should remain between the superficial fascia layers throughout it’s course, if the vein is seen to rise superficially above the fascia – mark this point and record the distance between the mark and the SPJ to determine the length of SSV segment that runs within the fascia. Measurements of the diameter of the SSV and depth of SSV from the skin should also be recorded. Additionally make a note of any unusual anatomy or the presence of thrombus in the SSV of Gastrocnemius veins recording still images as needed

LSV System

Image the long saphenous vein (LSV) in transverse view from just above the ankle level. If the LSV is present (dependent on previous surgery), check for reflux with colour Doppler using a distal compression and release. When LSV reflux is detected, follow the LSV in longitudinal or transverse view up the leg to the sapheno-femoral junction (SFJ) noting any filling from perforating veins, veins originating from the SPJ and the pudendal region. If the LSV appears competent or absent / removed surgically, image any visible incompetent surface veins, following these veins up the leg if possible to the site of incompetent filling. The LSV should remain between the superficial fascia layers throughout it’s course, if the vein is seen to rise superficially above the fascia – mark this point and record the distance between the mark and the SFJ to determine the length of LSV segment that runs within the fascia. Measurements of the diameter of the LSV and depth of LSV from the skin should also be recorded. Additionally make a note of any unusual anatomy, prominent branches or the presence of thrombus within the LSV system, recording still images as needed.

Finally, image the femoral vein below the SFJ in longitudinal view and check for deep venous incompetence using colour Doppler during distal calf compression and release.

Repeat the process for the other leg if this was requested.

Explain the findings of the examination in a manor appropriate to the individual patient.

Images should be reviewed and checked before ending the exam and then sent to PACS. As soon as it is practically possible, write up the findings of the investigations on CRIS. If specifically requested a hard copy of the report should be sent to the referring physician.