Extra-cranial cerebral artery assessment

Aim: To image the carotid and vertebral trees and identify the sites and severity and pathology of any arterial disease

Procedure:  Obtain a signed request from a clinician to perform the examination.

# Identify the patient by at least 2 points of reference (i.e. full name, d.o.b, address) and explain the procedure.  Ensure the patent is suitably dressed and positioned to proceed with the examination, positioning is usually supine with head supported by a pillow but other positions may be require depending on circumstances.

Using the appropriate scanner, probe and setting, image the right common carotid artery (CCA) in longitudinal view. Note the presence of atheroma, using colour Doppler to highlight lesions if appropriate. Record the peak systolic and peak end diastolic velocity from mid lumen of mid CCA. Care should be taken to ensure the angle of insonation is accurately measured from the vessel lumen (or any high velocity jet), acute angles (>65%) should be avoided if possible by adjusting the probe position. Moving the probe through the bulb and identify the carotid bifurcation and note the presence of any atheromatous disease. Using spectral Doppler confirm the patency of the internal carotid artery (ICA) and the external carotid artery (ECA). Record the peak systolic and peak end diastolic velocity from mid lumen of ICA and ECA just beyond the origin, or in the case of disease immediately beyond the point of the apparent maximum stenosis as highlighted using colour Doppler. Image any lesions in transverse and longitudinal views to help estimate the diameter reduction and the nature of the lesion based on the pattern of echogenicity. Images of any lesions, particularly of the ICA should be recorded, along with measurements of length of the diseased segment.

The severity of disease in the ICA should be estimated using diagnostic criterion as shown in Table (A) in the Appendix section. These diagnostic criteria should not be used in isolated but as part of a holistic diagnosis process based, in addition, on the visible diameter reduction and external factors such at the severity of any contra-lateral disease.

Moving the probe laterally the vertebral artery should be imaged in longitudinal view between the transverse processes of the vertebrae. The vertebral artery should be checked for patency using spectral Doppler and any abnormalities in flow pattern or flow direction noted.

The examination process should be repeated for the contra-lateral side.

Explain the findings of the examination in a manor appropriate to the individual patient.

Images should be reviewed and checked before ending the exam and then sent to PACS. As soon as it is practically possible, write up the findings of the investigations on CRIS. If specifically requested a hard copy of the report should be sent to the referring physician. When disease in excess of 70% is noted in a patient with appropriate carotid territory symptoms a copy of the results of the examination should passed on to one of the Vascular Surgeons regardless of the source of the original referral.

References

Peripheral Vascular Ultrasound How When and Why. Thrush and Hartshorne. Chapter 8.

Guidelines of Working Practice for the Vascular Scientist.

Joint Recommendations for Reporting Carotid Ultrasound Investigations in the United Kingdom.  C.P. Oates, A.R. Naylor, T. Hartshorne, S.M. Charles, T.Fail, K. Humphries, M. Aslam, P. Khodabakhsh. Diagnostic guidelines.

APPENDIX

Table 1 Diagnostic criterion to be applied.

| Percentage stenosis (NASCET) | Internal carotid peak systolic velocity cm/sec | Peak systolic velocity ratio ICAPSV/CCAPSV | St Mary’s ratioc ICAPSV/CCAEDV |
| --- | --- | --- | --- |
| <50 | <125a | <2b | <8 |
| 50-59 | >125 | 2-4a | 8-10 |
| 60-69 |  |  | 11-13 |
| 70-79 | >230 | >4 | 14-21 |
| 80-89 | >400b | >5b | 22-29 |
| >90 but less than near occlusion Occlusion | High, low-string flow | Variable Not applicable | >30 Variable Not applicable |

aNACC

bFilis et al

c Nicolaides et al