## Department of Vascular Surgery

## Broomfield  Hospital Vascular Lab

## Mid Essex Hospital Services NHS Trust

## Lower Limb Arterial Duplex Protocol to be utilised by all Sonographers/Scientists trained in Arterial Duplex imaging and reporting.

## History Taking:

## 1.Confirm referral details and identification with the patient prior to the start of the examination. Confirm patients symptoms and take any additional relevant information that you may require e.g. Diabetes, smoking, previous PVD, Heart/stroke history, blood pressure, cholesterol, claudication distance, rest pain etc.

## 2. Inform patient of examination that you are going to undertake and ask for removal of any garments that you will require.

## Patient preparation:

## 1.Removal of garments of limb(s) to be assessed.

## 2.Patient supine on the couch with a pillow for head support, can be sat up slightly if need to be due to respiratory disorders/musculoskeletal complaints. Leg to be assessed turned slightly outwards with a slight bend in the knee. Paper towel should be provided at the groin level to prevent gel transfer onto patients’ undergarments. In addition to this a sheet should be provided to cover the other limb to maintain the patient’s dignity.

## 3. Appropriate arterial scanning preset on the ultrasound machine and a linear 8-4 MHz probe selected. Gel should be applied to the area to be scanned or on the probe. Alternatively in large or slim patient a lower or higher frequency probe can be utilised.

## Scanning protocol:

## Starting at the level of the groin in transverse section identify the CFA, PFA and SFA in B-mode and in colour Doppler.

## Waveform analysis of the CFA, PFA and SFA origins should be performed with the use of colour and spectral Doppler in longitudinal section at 60 degree angles or less. If the CFA waveform is abnormal and iliac disease is suspected then this scan should be aborted and an aorto-iliac scan undertaken.

## Proceeding down the medial aspect of the leg, in longitudinal section, the entire SFA should be imaged in colour Doppler with spectral waveform analysis undertaken at minimum in the mid and distal regions.

## If a stenosis is encountered demonstrated by visible narrowing/colour aliasing, then waveforms and PSV should be recorded, prior to, at and beyond the stenosis.

## If possible the patient should then be turned on to their side to visualise the popliteal artery. Image the entire popliteal artery taking a minimum spectral trace in the proximal, mid and distal regions.

## Image the popliteal into the TPT and ATA origin. Waveform analysis should also be assessed in both these areas.

## Return the patient to the supine position with the leg straight on the bed for examination of the crural vessels.

## Image the ATA from the lateral anterior aspect of the proximal calf, to ascertain a recurrent anterior tibial artery feed in longitundinal section. Assess the ATA  in it’s entirety making sure to take a distal vessel waveform analysis.

## The PTA can be imaged from the medial aspect of the distal calf, by the medial malleolus (ask patient to roll leg out slightly). Identify the PTA and it’s veins and in longitudinal section assess the artery in its entirety back to the TPT in colour Doppler (waveform analysis at distal calf required)

## The Peroneal can be assessed from the same aspect as the PTA, it is slightly deeper that the PTA in this view. Starting at the ankle, image the vessel in its entirety back to the TPT with colour Doppler (waveform analysis at the distal peroneal is required).

## If disease is encountered, velocity measurements are utilised to grade the stenosis (see Table 1.1), and waveform shape are also required to detect the severity of the disease. Disease should be noted, along with the extent, severity and length. Collateralisation should also be noted. Anatomical variation should be reported.

## A written report of the results on the CRIS/ICE system should be performed, with the results being made available to the referring consultant.

## If patients symptoms have changed since the referral of the patient  initially and are now worse in as much as significant new rest pain/necrosis or ulceration of the foot and leg are apparent, one of the vascular team needs to be made aware  (+/-) examination of the patient, prior to the patient leaving the hospital.