UNIVERSITY HOSPITALS OF COVENTRY AND WARWICKSHIRE

VASCULAR LABORATORY

PROTOCOL FOR ASSESSMENT OF LEG VENOUS  INSUFFICIENCY

November 2008

INTRODUCTION

Venous insufficiency due to valvular incompetence can lead to varicose veins, skin changes (lipodermatosclerosis), haemosiderin pigmentation at the ankles and venous ulceration.

The location and  severity of venous reflux needs to be assessed to help decide whether the patient may benefit from surgery, foam sclerotherapy injection or other endovenous ablation techniques. Usually surgical procedures are performed when there is isolated superficial venous.

Information needs to be given as to whether any reflux is present in the deep veins, superficial veins or perforators.

HISTORY

Ask the patient if they have had a previous DVT and if so, when and in which leg.

Ask the patient if they have had any previous venous surgery or other venous procedures such as sclerotherapy injections and ascertain the site of the surgery. Also ask the patient where their varicose veins are.

PREPARATION

Any compression bandages must be removed.

Cover any ulcers with cling film if necessary.

Currently we do not usually scan over ulcerated areas, unless specifically requested to by the Vascular surgeons.

VALSALVA MANOEUVRE

The Valsalva manoeuvre involves the patient taking a deep breath and then bearing down on their abdomen with their breathing and abdominal muscles whilst retaining their breath. Another alternative explanation of the manoeuvre is to get the patient to take a deep breath and hold their nose and try to make their ears pop.

This is used to help estimates the state of the abdominal veins.

REFLUX

Reflux is assessed by squeezing and quickly releasing usually either the calf or foot.

Ideally a velocity of >30 cm/s should be achieved during the squeeze.

Sometimes this is performed in transverse projection, but more usually in longitudinal projection with colourflow Doppler and pulsed Doppler.

Reflux time can be quantified on Pulsed Doppler. A reflux time of more than 0.5 sec. is thought of as significant. Note if the reflux is of low velocity (<10 cm/s) in which case the reflux is usually considered mild.

METHOD

ASSESSMENT OF DEEP VEIN PATENCY

With the patient supine using the lower limb venous preset and L7 linear probe.

Visualise the CFV at the groin first using a transverse projection and then changing to a longitudinal projection observe the phasicity of the flow in pulsed mode.

If the flow is low and flat then it is possible that the iliac veins may be obstructed.

Ask the patient to perform a Valsalva manoeuvre and note the flow in the CFV.

If the deep veins are patent then their should be cessation of flow or reverse flow in the CFV and the CFV should be seen to expand.

Return to a transverse projection and compress the CFV and every 1 - 2cm down the SFV looking for complete compression. In the adductor canal it may be useful to compress from a posterior aspect with the other hand. It may also be useful to use the abdominal probe in the adductor canal.  If the SFV cannot be seen in transverse projection then longitudinal projection and compression to augment flow may allow the SFV patency to be evaluated.  Comment on any evidence of scarring (venous wall thickening) or any signs or old thrombus within the veins.

From a posterior  projection repeat the compression in transverse behind the knee for the the popliteal artery.

Usually  PT and Peroneal veins patency are assessed in longitudinal or transverse from a medial projection in mid calf using colour Doppler and augmentation by compressing the distal calf. Sometimes this is better left to when assessing reflux in the tibial veins when standing when the veins are more distended as more easily seen.

ASSESSMENT OF VENOUS  INSUFFICIENCY

This is normally performed with the patient standing. This is easiest for the operator if the patient stands on a step with a hand support. Alternatively if this is not possible, then the patient may stand on the floor resting either on the couch behind them with or without the support of a zimmer frame.

If the patient cannot stand then it is sometimes possible to perform reflux studies with the patient sat on the edge of a couch. The thigh must be angled downwards by at least 30 degrees.

Warn the patient that if they become light headed to let us know – so that they can sit down for a rest to recover before proceeding.

The patient should have most of the weight on their contralateral leg. The leg to be scanned should be bent slightly and rotated laterally with the muscles as relaxed as possible.

Use the Lower limb venous preset and the L7 linear probe.

In transverse projection locate the CFV at the groin above the SFJ, then use a longitudinal projection.  Compress and release the calf whilst insonating the CFV with Colourflow Doppler and  Pulsed Doppler and record any reflux.

Next assess reflux in longitiudinal projection in the SFV a few cm from its origin and record any reflux.

In tranverse projection find the SFJ and assess any reflux using colourflow. Note if this originates from the LSV if present, recurrent veins or from veins originating in the pelvis.  Alsp assess reflux in longitudinal projection in the LSV a few cm from the junction with colourflow and Pulsed Doppler.

Next, follow the LSV in transverse projection down the leg medially, periodically assessing reflux, noting where the main varices originate from and noting and assessing reflux, position relative to the knee or malleolus and diameter in any significant perforators (those > about 3 mm diam.). Note that perforators may originate from branches of the LSV.

In longitudinal, then if necessary transverse projection assess reflux in the PT and peroneal veins using colourflow and pulsed Doppler.

Ask the patient to face away from you to examine the popliteal fossa.

The patient should have most of the weight on their contralateral leg. The leg to be scanned should be bent slightly with the muscles as relaxed as possible.

In transverse projection locate the SSV and find the junction if possible.

Assess any reflux at the junction using colourflow and pulsed Doppler.

Note the height of the SPJ relative to the knee joint.

Note any reflux in the thigh extension of the SSV if present.

Note if the SSV has a common trunk with the gastrocnemius veins.

Follow the SSV down the calf  looking for any perforators or varices emerging from the SSV.

Next assess reflux in the popliteal vein below the SPJ.

Also assess reflux in the popliteal vein above the SPJ (if present).

Note that helical flow in the popliteal vein can sometimes give a false positive result for reflux.

Attempt to follow the major groups of varices back to their origin.

C.Tiivas. 2008.

REFERENCES

1. Vascular Laboratory Practice, Volume 3, Lower Limb venous assessment, 2001, IPEM/ Society for Vascular Technology for Great Britain and Ireland.

2. Peripheral Vascular Ultrasound, How, Why and When, A. Thrush and T.Hartshorne, 2nd Edition, 2005, Chapter12.