| Purpose of SOP | To ensure standard practice of all staff undertaking Duplex Ultrasound scanning carried out to assess for the presence of pathology and the haemodynamic status of the common carotid artery (CCA), Internal carotid artery (ICA) external carotid artery (ECA) and vertebral artery. |  |  |
| --- | --- | --- | --- |
| Average Time to Operate | Full scan including booking on to Meditech and reporting 30 minutes | Frequency of Use | Daily |
| Project/System | Diagnostic ultrasound | Owner |  |
| Tools / Equipment | Colour Duplex Ultrasound machine with Vascular presets, linear (5 MHz) and Curvi-linear (3-3.5 MHz) transducers, ultrasound gel, Height adjustable, preferably electrical examination couch, chair, pen. The examination room should be temperature controlled with adjustable lighting levels suitable for examination. Suitable cleaning materials should be available in line with local and manufactures guidelines |  |  |

| No | Main Operating Steps | Key Points | Explanation / Examples / Diagrams |
| --- | --- | --- | --- |
| 1 | Apply standard infection control precautions and all relevant Health and Safety measures | Ensure patient/relative/carer understands the procedure and obtain verbal consent as per Trust procedure | Also see Vascular Laboratory Practice (IPEM) Part II |
| 2 | It is important to follow the same scanning sequence to avoid missing important information |  |  |
| 3 | Patient positioning | Recommended scanning position: Patient on examination couch with head slightly raised or sat upright in a chair with head slightly tilted. |  |
| 4 | Scan preparation | The patient is asked to adjust their clothing to expose the neck area. Tuck tissue paper into top to prevent soiling of clothing with gel. Apply Ultrasonic Gel to U/S probe. |  |
| 5 | Imaging of Carotid and Vertebral arteries | The patient is examined in the supine position with their head/neck positioned in such a manner that allows for maximum access to the vessels to be examined. Bilaterally arterial supply to the head should be assessed, encompassing of the: - Common carotid artery (CCA) - Carotid bifurcation - External carotid artery (ECA) - Internal carotid artery (ICA) to its most accessible distal extracranial segment. - The vertebral artery (VA) should be identified to confirm direction of flow. In the presence of reversed or partially reversed flow the subclavian artery should be examined. The CCA, carotid bifurcation, ECA and ICA are identified in B Mode using the transverse plane and longitudinal plane; B-mode can be used to classify echogenicity of any plaque and the surface characteristics e.g., echolucent, mixed, dense, calcified irregular, smooth or ulcerated. Using longitudinal plane with colour and spectral Doppler (angle of <60 degree), the extracranial carotid arteries should be assessed for any areas for velocity increase or turbulence. Peak systolic velocities (PSV) and end diastolic velocities (EDV) should be measured and documented for a minimum of the CCA and ICA. The anatomical location of any haemodynamically significant lesion should be documented. In patients where the U/S is performed as part of a pre-op work-up it is only necessary to scan the side of planned carotid endarterectomy, and where request mark the location of the carotid bifurcation on the patient’s skin using a surgical maker. |  |
| 6 | Carotid stenosis criteria | The anatomical location of any haemodynamically significant lesion should be documented using the following criteria: Percentage stenosis (NASCET); ICA Peak systolic velocity (PSV) cm/sec; ICA End systolic velocity (EDV) cm/sec; Peak systolic velocity ratio ICA PSV /CCA PSV; St Mary’s Ratio ICA PSV /CCA EDV. <50; <125; <40; <2; <8. 50-59; >125; 40-100; 2-4; 8-10. 60-69; 11-13. 70-79; >230; >100; >4; 14-21. 80-89; 22-29. >90 but less than near occlusion; >400; >5; >30. Near occlusion; High, low-string flow; Variable; Variable. Occlusion; No Flow; Not applicable; Not applicable. Where possible dimeter reduction measurements can be made in accordance with NASCET method. Dimeter reduction measurements made in the bulb should be made using NASCET method to correlate with velocity criteria. |  |
| 8 | Report | The report to be competent on Carestream PACS and should be competent by the person undertaking the examination. The report should include correct patient demographics, date of examination, examination type and name and status of person reporting the examination. The report should include: - Which arteries have been assessed and the presence / absence of disease - Plaque characterises (soft, mixed, dense calcified, smooth, irregular) and length of lesion should be documented. - Percentage degree of stenosis and method used to obtain degree of stenosis. - If diameter reduction measurements name state what method ECST or NASCET (recommended) - Patency and direction of flow in both vertebral arteries should be included. - Where the vessel is obscured, then length of obscured region should be noted. - Any limitations e.g., acoustic showing due to calcified plaque, high bifurcation, short neck. - Where appropriate suggestion for alternative imaging may be recommended - Where appropriate suggestion for Vascular opinion may be recommended |  |
| 9 | Referral / Booking Pathway | Two/ three dedicated TIA pm slots for the Urgent TIA clinic patient are available daily, these are labelled on TOMCAT as TIA Urgent. If there are no patients booked for this clinic or they do not require scanning, then the slots are to be used for A/E or IP Carotid request can be made for urgent outpatient appointments. |  |
| 9 | Further Action to be acted upon post scan. | Critical ultrasound results should be made aware to the referring consultant/ TIA clinic / stroke coordinators via phone or email. |  |
| 10 | END OF OPERATION |  |  |