| Purpose of SOP | To ensure standard practice of all staff undertaking Duplex Ultrasound scanning for the diagnosis of arterial insufficiency relative to Intermittent Claudication, critical ischaemia, rest pain, tissue loss as well as routine surveillance post-intervention: graft patency, angioplasty, and stenting. |  |  |
| --- | --- | --- | --- |
| Average Time to Operate | - Full scan of 1 leg and reporting 30 minutes - Full scan of 2 legs and reporting 60 minutes - Full scan of Aorto-iliac arteries and 2 legs and reporting 90 minutes | Frequency of Use | Daily |
| Project/System | Diagnostic ultrasound | Owner |  |
| Tools / Equipment | Colour Duplex Ultrasound machine with Vascular presets, linear (9MHz Imaging, 3 MHz Doppler) and Curvi-linear (5 MHz Imaging, 1 MHz Doppler) transducers, ultrasound gel, adjustable examination couch, adjustable chair, skin marker, tape measure and pen. |  |  |

| No | Main Operating Steps | Key Points | Explanation / Examples / Diagrams |
| --- | --- | --- | --- |
| 1 | Apply standard infection control precautions and all relevant Health and Safety measures | Ensure patient/relative/carer understands the procedure and obtain verbal consent as per Trust procedure. |  |
| 2 | It is important to follow the same scanning sequence to avoid missing important information |  |  |
| 3 | Patient positioning | Recommended scanning position: Arterial Duplex scans are best performed in a supine/semi-reclined position on an examination couch. Common femoral (CFA), proximal profunda femoris (PFA) and superficial femoral (SFA) arteries: leg externally rotated, and knee slightly flexed. Popliteal – as above in patient with good hip mobility (probe positioned behind knee) or patient positioned in lateral decubitus position, so leg is turned to the side and probe positioned to the back of knee. Calf arteries – a combination of those positions described above (for Anterior tibial (ATA), Posterior tibial (PTA) arteries as for femoral investigations, whereas for peroneal artery as that for popliteal artery investigation or with knee bent at 45-degree angle. |  |
| 4 | Scan preparation | Patient to remove necessary clothing. Offer patient a gown to maintain patient dignity. Ask patient to tuck tissue paper into clothing to prevent soiling of clothing with gel. Apply Ultrasonic Gel |  |
| 5 | Imaging of arteries | As a standard the following should be assessed as assessment of Lower limb arteries - Common femoral artery (CFA) - Proximal profunda femoris artery (PFA) - Superficial femoral artery (SFA) - Popliteal artery (POP A) - Tibio-peroneal trunk (TPT) are any VRO identified. - Posterior tibial artery (PTA) - Anterior tibial artery (ATA) - Peroneal artery (Per A) On request from a referring team, or in the event of an abnormal waveform (e.g., reduced monophasic waveforms) in the CFA, possibly due to obstructive disease in a proximal artery, an examination of the aorto-iliac arteries may be necessitated, including: Abdominal - Aorta - Common iliac artery (CIA) - External iliac artery (EIA) The ideal position for an aorto-iliac artery investigation is supine, to allow full extension of the spine and thus the arteries to prevent positional kinking. A curvilinear transducer is employed. When imaging the aorta and iliac arteries, look for any sizes of aneurysms. If there is a suspicion of aneurysmal disease, then the AP diameters should be measured in cm (see AAA SOP for more detail). Upper arm - Common carotid artery (CCA) - Subclavian artery (Sub A) - Axillary artery - Brachial artery (BA) - Radial artery - Ulnar artery Grafts / Stents - Inflow to the graft / stent (vessel proximal to graft) - Proximal anastomosis / stent origin - Entire length of graft / stent - Distal anastomosis / stent termination - Outflow vessels (vessels distal to the graft/stent) The following technique should be applied to assess lower limb and upper limb arterial systems: - B-mode should be used to image the artery and assess for aneurysmal dilations, and nature of luminal content e.g., plaque, thrombus, smooth, irregular, diffuse, dense calcified. - Colour Doppler should be used to assess for the presence / absence of flow and aid the position of spectral Doppler when quantifying a stenosis. The colour Doppler and steer box should be aligned in the correct direction in accordance with vessel orientation. - Switch on spectral Doppler and align sample volume to centre/midstream vessel and determine direction of flow, measurement of velocity. Steer the Doppler beam so that the line of sight is parallel to the vessel walls. Fine-tune Doppler angle using angle correction feature, ensuring Doppler beam to vessel angle is 60 degrees or less. - Activate spectral Doppler via “update” control, and switch On High-Q, which tracks the envelope of the Doppler signal, providing velocity (peak systolic velocity or PSV), waveform sub-type (triphasic, biphasic, monophasic) and pulsatility index for the CFV (greater or lower than 6.0) data. Ensure at least 4-5 cardiac cycles are recorded on the spectrogram. Also note any qualitative changes to the waveforms such as irregularity of rhythm to indicate cardiac abnormalities or spikes to indicate turbulence. |  |
| 6 | Diagnostic criteria | In the case that atheroma is present, the following should be noted. Qualitative data for B-mode, Colour Doppler and Pulse wave velocity includes: For spectral Doppler this may include cardiac arrhythmias and turbulence, but always the type of waveform: triphasic (normal), biphasic (good), monophasic (poor) as well as hyperaemic which indicates high peripheral vasodilation elicited by several physiological states including infection, or a thump signal suggestive of an impending obstruction. - Location and length of plaque (e.g., SFA mid-thigh) - Severity of (mild, moderate severe, stenosis, occlusion), focal disease or multi-level disease or an occlusion - Plaque morphology (echolucent, mixed, dense, calcified, smooth, irregular, ulcerated). - Length of the disease can be measured using callipers and recorded on the images. - Use of colour Doppler to show absence of colour signals and determine direction of flow (antegrade / retrograde flow). - Spectral Doppler to determine type of waveform: triphasic (normal), biphasic (Good), monophasic (Poor) as well as hyperaemic which indicates high peripheral vasodilation elicited by several physiological states including infection, or a thump signal suggestive of an impending obstruction. Cardiac arrhythmias and turbulence may also be observed. - Presences of any collateral vessels. - Any anomalies such as dissection intimal flaps, cystic adventitial disease, aneurysm. Quantitative data is largely based on flow velocity information, the main criterion used to grade a degree of narrowing in the artery is the peak systolic velocity ratio (PSV. All increases in flow velocity at a diseased segment should be compared to the flow velocity, in an adjacent proximal segment of normal vessel. From here the PSV ratio can be calculated which relates to the level of disease as follows: PSV ratio; Severity of Disease. <2; Mild disease / stenosis (stenosis <50%). ≥2 - <4; Moderate stenosis (50-75% reduction in diameter). ≥4; Severe stenosis (>75% reduction in diameter). Absence of PWV; Occlusion. The PI in the CFA can be used as a surrogate measure of iliac artery patency. A normal CFA PI is >/= 6.0, whereas an abnormal CFA PI is < 6.0. This index should be included in the report to guide consultants in the event of planning for endovascular/surgical interventions. Furthermore, diameter and area reductions (residual diameter/true diameter X 100) of localised plaque using B-mode and/or colour Doppler can be used to complement velocity data, to demonstrate degree of stenosis. However, caution should be applied using this technique given the 2-D imaging of a 3-D artery, and over-estimations are likely in the presence of eccentric atheroma. |  |
| 7 | Report | The report to be competent on Carestream PACS and should be competent by the person undertaking the examination. The report should include correct patient demographics, date of examination, examination type and name and status of person reporting the examination. The report should include: - A description of each artery should be provided in the report, whether patent and if diseased. - If disease the anatomical position, length and plaque morphology should be given. - The severity of the disease should be given, PSV ratio, or changes in PSV. - The anatomical position and size of any aneurysms. - Comment of pattern of the Doppler waveforms at different location. - Any limitation to the assessment (body habitus, oedema, acoustic shadowing, calcified vessels, bowel gas, overlying dressings). - Were scans are considered non-diagnostic, recommendations for further alternative imaging should be recommended. Where investigation is for a surveillance or follow up scan of an intervention, comment should be made comparing result to previous assessment if they are accessible. |  |
| 8 | Scan limitation | Scan limitations include large patient habitus, overlying oedema, ulceration +/- dressings, patient intolerance to probe pressure, bowel gas, arterial calcification, shadowing from scar tissue. On such occasions these limitations must be clearly stated in the report and an indication as to whether further alternative imaging would be required as to provide adequate diagnostic information for potential treatment. |  |
| 9 | Further Action to be acted upon post scan. | For those patients who have been referred for routine investigations due to stable conditions can leave the CRV dept with the instructions that the consultant will contact them with a date and time of follow-up in OP clinic to discuss results and potential treatment options. ### For those patients who have been referred for urgent investigations due to critical ischaemia or tissue loss, it may be that communication with the referring consultant team or vascular on-call team is required before patient leaves CRV, especially if symptoms have deteriorated. Alternate the report can be emailed to the Vascular secretaries at the email address below for an urgent review. coc-tr.vascularsecretaries@nhs.net |  |
| 26 | END OF OPERATION |  |  |