| Purpose of SOP | To ensure standard practice of all staff undertaking Duplex Ultrasound scanning for the evaluation the deep and superficial venous systems for evidence of valvular incompetence and to establish the source of any reflux identified in the superficial veins |  |  |
| --- | --- | --- | --- |
| Average Time to Operate | Full scan including booking on to Tomcat / Cerna and reporting 30 minutes per limb | Frequency of Use | Daily |
| Project/System | Diagnostic ultrasound | Owner |  |
| Tools / Equipment | Duplex Doppler ultrasound machine with imaging frequencies of 3.5MHz and greater with colour Doppler capability; with both linear and curvilinear transducers available. The examination couch should be height adjustable, preferably electrical. The couch should be capable of tilting by at least 30 degrees. The CVS’s chair should provide good lumbar support, be height adjustable and allow for the CVS to move close to the examination couch. Skin pen marker. Measuring Tape. Foot stool, preferably heigh adjustable. Paper for drawing. The examination room should be temperature controlled with adjustable lighting levels suitable for examination |  |  |

| No | Main Operating Steps | Key Points | Explanation / Examples / Diagrams |
| --- | --- | --- | --- |
| 1 | Apply standard infection control precautions and all relevant Health and Safety measures | Ensure patient/relative/carer understands the procedure and obtain verbal consent as per Trust procedure | Also see Vascular Laboratory Practice (IPEM) Part IV |
| 2 | It is important to follow the same scanning sequence to avoid missing important information |  |  |
| 3 | Patient positioning Venous Duplex scans are best performed on a tilting couch, approx. 20-30o tilt. | Recommended scanning position: Femoral veins - supine (reverse Trendelenburg), leg externally rotated, and knee slightly flexed. Popliteal - supine or with patient sitting on the couch with legs dependent. Calf veins - with patient sitting on the couch with legs dependent and feet placed on a foot stool. |  |
| 4 | Scan preparation | Patient to remove trousers/clothing/ compression stockings. Placement of towel/gown/sheet across lap to maintain patient dignity. Ask patient to tuck tissue paper into underwear to prevent soiling of clothing with gel. Apply Ultrasonic Gel to inner thigh to knee. |  |
| 5 | Imaging of lower-limb veins | The following techniques should be applied to all venous segments: B mode should be utilised to assess vein patency by observation of the compressibility of the vein, this should be done in a transverse scan. Pulsed and colour Doppler should be utilised to assess flow characteristics within the veins, this will include assessment of phasicity, spontaneity and direction of flow. Flow characteristics will generally be assessed in a longitudinal scan plane. Manual automated distal augmentation should be used to enhance the flow and to assess for reflux. Incompetence (defined as a reflux time >0.1s) or slight incompetence (reflux >0.5s to <1.0s) should be noted. If thrombus or superficial thrombophlebitis (STP) is identified the extent of the thrombus / STP should be noted. Start the examination in the groin at the common femoral vein (CFV) and assess the Compressibility, phasicity and flow direction. Flow should be spontaneous with respiratory and cardiac modulation. Abnormal flow in the CFV or abnormal superficial veins in the groin/abdomen may be due to iliac vein obstruction and in these cases the iliac veins and inferior vena cava need to be examined. Continue to examine the lower limb deep veins distal to the CFV, examining the length of the superficial femoral vein (SFV), the proximal profunda femoris vein and the popliteal vein as detailed above. Once the deep veins have been assessed the transducer should be moved back to the groin to assess the sapheno-femoral junction (SFJ) and long saphenous vein (LSV). The LSV should be assessed throughout its length for patency and competency. The location and extent of any incompetent segments should be noted, along with the position of any associated significant / incompetent perforators or branches. It may be useful to record the diameter and depth of the vein if endovenous treatment may be considered. The deep veins in the calf should also be examined. The patient should be repositioned to assess the patency and competency of the sapheno-popliteal junction (if present) and the short saphenous vein (SSV). The anatomy associated with the origin of the SSV is very variable and should be commented on if the short saphenous vein is incompetent or if varicose veins arise from this area. Any varicose veins that have not been linked to either the long or short saphenous system should also be examined to identify any other sources of reflux – i.e., incompetent perforators. |  |
| 6 | Report | The report is a recording and interpretation of observations made during the lower limb venous duplex ultrasound examination; it should be written by the CVS undertaking the examination and viewed as an integral part of the whole examination. The report should include correct patient demographics; date of examination; examination type and the name and of the CVS. The report should include: - The presence/absence of phasic flow in the CFV - Which veins have been assessed, the competency of the veins, the extent of incompetent segments, the presence/absence of any thrombus / STP - Any anatomical variations due to previous procedures (i.e., absence of LSV due to previous strip) - Where thrombus / STP is identified, the location, length/extent, degree of patency and estimated age should be documented. - Any limitations e.g., if areas in the calf are not visualized due to ulceration. - An appropriate number of annotated images that represent the entire ultrasound. - examination - in accordance with local protocols and SVT Image Storage Guidelines Report should include all clinically important findings. As already noted, any limitations of the scan should be recorded and where scans are considered non-diagnostic, recommendations for further alternative imaging should be recommended. |  |
| 7 | Further Action to be acted upon post scan. | If a patient is found to have an acute DVT or acute STP on a routine reflux study, contact the DVT nurses and ask if they can review patient. If they cannot then contact the Vascular on call team and ask them to review patient. For those patients who have been referred for IP reflux investigations, these patients will be returned to their wards following their scan, report will be accessible on Carestream and / or Cerna. |  |
| 8 | END OF OPERATION |  |  |