Extra-cranial Cerebral Artery Assessment

Aim: To image the carotid and vertebral trees and identify the sites and severity and pathology of any arterial disease

Procedure:  Obtain a signed request from a clinician to perform the examination.

# Identify the patient by at least 2 points of reference (i.e. full name, d.o.b, address) and explain the procedure.  Ensure the patent is suitably dressed and positioned to proceed with the examination, positioning is usually supine with head supported by a pillow but other positions may be require depending on circumstances.

Using the appropriate scanner, probe and setting, image the right common carotid artery (CCA) in transverse view and follow the vessel up through bifurcation level and into the ICA and ECA segments. Note the presence of atheromatous disease, using colour Doppler to highlight lesions if appropriate.

Image the CCA in longitudinal view and record the peak systolic and peak end diastolic velocity from mid lumen of distal CCA. Care should be taken to ensure the angle of insonation is accurately measured from the vessel lumen (or any high velocity jet), acute angles (>60%) should be avoided if possible by adjusting the probe position. Moving the probe through the bulb and identify the carotid bifurcation and document the presence of any atheromatous disease.

Using spectral Doppler confirm the patency of the external carotid artery (ECA) by recording the peak systolic and end diastolic velocity from the mid lumen just beyond origin or in the case of disease immediately beyond the point of apparent maximum stenosis as highlighted using colour doppler.

Using spectral Doppler confirm the patency of the internal carotid artery (ICA) by recording the peak systolic and end diastolic velocity from the mid lumen within the proximal ICA segment beyond the bulb or in the case of disease immediately beyond the point of the apparent maximum stenosis as highlighted using colour doppler. Image the ICA with colour doppler up to the level of the mandible in longitudinal view, ensuring distal patency is maintained and record a spectral trace within the ICA distal to any significant disease.

Image any lesions in transverse and longitudinal views to help estimate the diameter reduction and the nature of the lesion based on the pattern of echogenicity. Images of any lesions, particularly of the ICA should be recorded, along with measurements of length of the diseased segment if haemodynamically significant.

The severity of disease in the ICA should be estimated using the NASCET grading criteria as shown in Table (A) in the Appendix section. These diagnostic criteria should not be used in isolated but as part of a holistic diagnosis process based, in addition, on the visible diameter reduction and external factors such at the severity of any contra-lateral disease.

Moving the probe laterally the vertebral artery should be imaged in longitudinal view between the transverse processes of the vertebrae. The vertebral artery should be checked for patency using spectral Doppler and any abnormalities in flow pattern or flow direction noted.

The examination process should be repeated for the contra-lateral side.

Explain the findings of the examination in a manor appropriate to the individual patient.

Images should be reviewed and checked before ending the exam and then sent to SECTRA PACS. As soon as it is practically possible, write up the findings of the investigations on SECTRA PACS.

References

Peripheral Vascular Ultrasound How When and Why. Thrush and Hartshorne. Chapter 8.

Guidelines of Working Practice for the Vascular Scientist.

Joint Recommendations for Reporting Carotid Ultrasound Investigations in the United Kingdom.  C.P. Oates, A.R. Naylor, T. Hartshorne, S.M. Charles, T.Fail, K. Humphries, M. Aslam, P. Khodabakhsh. Diagnostic guidelines.

APPENDIX

Table A Diagnostic criterion to be applied.

| Percentage stenosis (NASCET) | Internal carotid peak systolic velocity cm/sec | Peak systolic velocity ratio ICAPSV/CCAPSV | St Mary’s ratioc ICAPSV/CCAEDV |
| --- | --- | --- | --- |
| <50 | <125a | <2b | <8 |
| 50-59 | >125 | 2-4a | 8-10 |
| 60-69 |  |  | 11-13 |
| 70-79 | >230 | >4 | 14-21 |
| 80-89 | >400b | >5b | 22-29 |
| >90 but less than near occlusion Occlusion | High, low-string flow | Variable Not applicable | >30 Variable Not applicable |

aNACC

bFilis et al

c Nicolaides et al