BYPASS GRAFT FOLLOW-UP PROTOCOL

Review –  2 yearly

| Date | Version | Amendments made |
| --- | --- | --- |
| Oct 2014 | 1 | Protocol created |
| Oct 2015 | 1 | Protocol reviewed, no changes |
| 17/08/2016 | 2 | Protocol reviewed, referencing expanded following UKAS visit |
| 8/11/2017 | 3 | Following discussion in team meeting, agreed to report significant results to Vascular Radiologists and interdepartmental mailbox only. Updated in reporting section. |
| 08/05/2018 | 4 | Reporting section, bullet pt 14. no longer require to email Radiologists. |
| 4.2.19 | 5 | Reference section developed. Additional of advice regarding anastamosis stenosis detection where there is calibre mis-match. (AC) |
| 27.11.20 | 6 | Additional info re volume flow added, references added for volume flow and additional information added re process for reinstating surveillance after angioplasty. |
| 24/02/21 | 6 | Protocol reviewed, no changes (EM) |
| 27/04/22 | 7 | Volume flow measurement removed as per vascular surgeons’ request (MB) |
| 27/02/2023 | 8 | Removed guidance to include volume flow measurements for cross-over grafts (EM) |

Preparation

- If there is a request form – check whether more than a standard scan is required. There may be a requirement to also fully assess inflow or outflow. Otherwise carry out a standard graft follow-up scan detailed below.

- Check any previous scans, interventions etc. so that you are fully aware of the history of the relevant graft.

- Measure the ABPI if appropriate/requested. Do not place a blood pressure cuff over the distal anastomosis of a graft as it could occlude the graft. ABPIs may be inaccurate if measured when a patient has significant leg oedema or known calcified vessels. Use cling film/plastic apron to cover open wounds and be wary of painful ulceration.

Technique

- Use the most appropriate probe (linear for most superficial grafts).

- Check the inflow, including assessment of the CFA, PFA and SFA as appropriate and dependent on graft location. paying particular attention to the vessel just above the proximal anastomosis and record a Doppler waveform from this position and note any narrowing. Where good inflow cannot be established and particularly where inflow has changed from a previous assessment, extend the scan proximally to identify any source of haemodynamically significant disease.

- Record the velocity through the anastomosis, adapting technique for any areas of turbulence, and look for any areas of narrowing.

- Scan throughout the length of the graft in a longitudinal plane, assessing the colour flow for areas of narrowing or flow disturbance. (Set the PRF to approx half the velocity within the graft and adapt as necessary).

- As you scan, record the Doppler waveform and velocity at intervals (e.g. proximal, mid and distal thigh). Record the maximum velocity in any areas of high velocity flow.

Velocity criteria

<180cm/s is considered normal.

180 - 350cm/s suggests 50-75% diameter reduction (dr)

>350cm/s suggests >75% dr

For a graft with a large lumen and/or low/high velocities throughout you may need to use velocity ratios:

x2 suggest 50% dr

Between x2 to 4 suggest 50-75%d.r

>x4 suggest >75%dr

- Assess the distal anastomosis for narrowing; remembering that flow through a wide graft’s junction with a smaller calibre native vessel will have a velocity increase due to the difference in calibres, rather than to stenosis. In cases of significant calibre mis-match, you can compare the velocity in the ?stenosis to the velocity in a non-diseased portion of the outflow vessel. Look for evidence of any narrowing in B-mode too. Ensure that velocity measurements are consistent with the appearance of the anastomosis.

- Record a Doppler waveform from the native vessel distal to the anastomosis and note any narrowing. Check the flow at the ankle if you think it would be appropriate (e.g. if the graft is occluded or threatened by severe stenoses or very low flow rates).

- For in-situ grafts and grafts where operation technique is not known , re-scan throughout the length of the graft in a transverse plane.  Use low colour PRF, looking for any side branches acting as fistulae, measure their diameter and assess their significance possibly measuring volume flows before and after the fistula.

- For popliteal aneurysm bypass grafts, also measure the A-P diameter (outer to outer) of the aneurysm. If there is an increase in size or colour flow within the excluded aneurysm, assess the native vessels to see where the filling is coming from. Any significant increase in size (use professional judgement) should be urgently reported to the Consultant.

- Minimally record spectral Doppler waveforms demonstrating inflow to graft, proximal anastomosis (PA) flow, proximal, mid and distal graft, distal anastomosis (DA) and outflow from graft. Also record images as appropriate demonstrating stenosis/significant narrowing.

- If there is a significant proximal stenosis take a Doppler waveform from the opposite groin, as you would do for an arterial Duplex, in view of possible future angioplasty via a contralateral approach.

Reporting

- Complete a diagrammatic paper report. Velocities only need to be added to the report if indicating areas of concern ie: graft stenosis or low velocity flow.

- If there is a significant stenosis with good flow distally and the graft does not appear immediately threatened email a copy of the report to the Vascular Surgeons and Vascular Nurse Specialists on:

VascularInterdepartmentalReferrals@porthosp.nhs.uk

- Note any significant finding on the VAU follow-up schedule form.

- Let the patient know that:

- You have found a narrowing in the graft

- You will alert the Vascular Surgeons

- they can expect to be contacted by the Vascular team who will decide whether an angioplasty is required.

- If there is a stenosis which may threaten the graft, or a recent (from history) graft occlusion:

- Keep the patient in the department.

- Contact the relevant Consultant/Registrar for advice.

- In the event of finding a significant stenosis book the patient’s next scan for 2 months time and note this on the report. Angioplasty is usually performed within two weeks and this will restart surveillance at approximately 6 weeks post plasty. VAU Admin staff will start a new line on the VAU follow-up schedule form leaving the Intervention details blank and schedule the next appointment for 2 months time. The Vascular Scientist completing the next scan must check CRIS/PACS for details of the Angioplasty and write a summary of the “Intervention” on the VAU follow-up schedule form

16. For longstanding occlusions, just inform the Consultant and Vascular Nurse specialist, as further follow-up would be inappropriate.

X-Over Graft Assessment

Follow relevant points above

- Check graft is patent

- Check direction of flow

- Record waveforms and velocities at proximal and distal anastomosis

- Grade stenoses using velocity criteria above

- Complete paper diagrammatic report

References

Stenosis criteria

The Society for Vascular Technology Professional Perfomance Guidelines https://www.svtgbi.org.uk/

Cronenwett JL, Wayne Johnston K (2014) Rutherford’s Vascular Surgery 8th Edn. Elsevier. (pg 247).

Zierier RE (2010) Standness’s Duplex Scanning in Vascular Disorders (pg 345)

Hutchinson SJ, Holmes KC (2011) Principles of Vascular and Intravascular Ultrasound. (pg102)

AbuRahma A, Bandyk D (2012) Non-invasive Vascular Diagnosis: A Practical Guide to Therapy. (pg 328).

Vascular Laboratory Practice Part III. “Lower limb arterial assessment” The Society for vascular Technology & Institute of Physics and Engineering in Medicine. 2001

Ihlberg LH et al. “Intraoperative flow predicts the development of stenosis in infrainguinal vein grafts” J Vasc Surg 2001; 34:269-76.

Tinder CN et al “Efficacy of Duplex ultrasound surveillance after infrainguinal vein bypass may be enhanced by identification of characteristics predictive of graft stenosis development” 2008 J Vasc Surg 48(3): 613-8

Polak, J et al “Early detection of Saphenous vein arterial bypass graft stenosis by colour-assisted Duplex sonography” Am J Roentgenol 1990 154:857-61

Calibre mismatch

Gerhard-Herman, M et al “Guidelines for Non-Invasive Vascular Laboratory testing: A report from the Americal Society of echocardiography and the Society of Vascular Medicine and Biology”  J Am Soc Echocardiogr 2006 19:955-72. https://www.asecho.org/wp-content/uploads/2013/05/Noninvasive-Vascular-Lab-Testing.pdf

Volume Flow measurement

Stranden, E., 2004. Methods for the Evaluation of Vascular Reconstruction. Heart Drug, 4(4): p201.

Hoyt, K. et al., 2009. Accuracy of Volumetric Flow Rate Measurements. Journal of Ultrasound in Medicine, 28(11): 1511-1518.