Carotids

These guidelines describe current local practice. They are intended to be an aid to best practice rather than to be prescriptive.

# Carotid and Vertebral Artery Duplex Protocol

Patient Identification

Ask the patient their full name, date of birth and\or address and check that these correlate with the request form (OUH Identification Policy Nov 2012).

If the patient is unable to communicate these details clearly, check inpatient wristband or ask accompanying relative/carer to confirm identity.

Patient Dignity

Ensure that the scanning environment is private and ensure the patients’ dignity at all times. Always ask permission from the patient for other people to watch the scan including medical students and other health professionals.

History

Take a brief history to ascertain relevant facts that will help support and direct the investigations such as date and duration and description of symptoms. If a stroke or TIA is suspected, ask the patient to point to the relevant side affected.

Explanation of Investigation

The scanning procedure and purpose of the scan should be explained to the patient in a clear and concise manner.

Consent

Patients must consent to the scan (see ORH trust policy for Consent to Examination or Treatment, June 2012). By attending an appointment it is assumed the patient is consenting to have a scan but this must be confirmed verbally. Discontinue scan if requested to do so.

Patient position

Ask patient to lie supine with head slightly tilted back rotated away from side being examined.

# Examination

B Mode

With the 9-3MHz linear array transducer, locate the common carotid artery (CCA) at the base of the neck. Follow the CCA, internal carotid artery (ICA) and external carotid artery (ECA) up the neck.

Examine the carotid arteries in a longitudinal and transverse plane. Image the vessels from several directions as necessary to best demonstrate any calcification or plaque, in particular at the carotid bifurcation.

Colour and Pulsed Doppler

- Repeat the process of assessing the vessels using Colour Doppler visualizing as proximally and distally as possible.

- Obtain Doppler waveforms in the distal CCA (1-2cm from bifurcation), ICA and ECA and at sites where pathology is seen or suspected, ensuring the highest velocities obtainable at each site are recorded. Peak systolic velocity (PSV) and end diastolic velocity (EDV) should be measured.  The length of disease (plaque length) in the carotid arteries, especially the internal carotid artery, should be recorded, as well as the morphology of plaques, possible dissections and any ulceration or unusual appearance.  Where major disease is seen in the proximal ICA, the size and patency of the distal ICA should also be recorded. The CCA reference velocity measurements should be taken from approx 1-2cm proximal to the bifurcation provided no significant disease is present at this location.

- Also comment on the level of bifurcation in the neck, length of plaque and presence of a patent distal vessel in any surgically significant diseased vessels.

- All velocity measurements should normally be made with a Doppler angled of 600 or between 45 and 60 degrees1.

- Any calliper measurements to also be labelled on the image with ECST or NASCET.

- Locate and obtain a waveform from the vertebral artery and note the direction of flow.

- If flow is retrograde or partially retrograde to the CCA flow, obtain a waveform from the subclavian artery and assess for proximal disease i.e. subclavian steal.  Note any abnormal flow signals/velocities.

# Image recording (reviewed July 2020)

In the interest of standardisation, the following is a list of the minimum images that should be recorded on each side and in which order –

- Proximal CCA (as close to origin as possible)

- ECA – colour and spectral Doppler

- Proximal ICA – colour and spectral Doppler

- Distal CCA – colour and spectral Doppler

- Distal ICA (velocity measurement not required) – colour and spectral Doppler to confirm patency

- Carotid bifurcation B-mode longitude

- Carotid bifurcation B-mode transverse

- Carotid bifurcation B-mode transverse with colour/power Doppler only if helpful in providing additional stenotic information but images MUST be optimised

- Vertebral artery – colour and spectral Doppler

- Additional images as considered necessary to demonstrate physiology/disease

(Any calliper measurements to also be labelled on the image with ECST or NASCET)

All images must be optimised and correctly labelled before saving.

# Examination Report

Complete the attached electronic report form colouring in any disease on the diagram.

Percentage Stenosis

Where possible, the ‘Joint Recommendations for Reporting Carotid Ultrasound Investigations in the United Kingdom’ should be used as the primary method of calculating a >50% ICA stenosis as per the table below. However, if an acceptable correlation cannot be obtained between the peak systolic velocity and one or more velocity ratios then the following may be used –

- A combination of peak systolic velocity and visual assessment using B-mode and colour Doppler.

- Calliper measurements using strict NASCET criteria.

The method used must be clearly stated on the report.

For ICA stenoses <50% that are not clearly supported by velocity ratios as shown on the table, visual assessment or calliper measurements of diameter reduction can be used as per the diagrams below. The method used must be indicated on the report as these measurements do not follow NASCET criteria.

# Assess percentage stenosis in the internal carotid artery (ICA) using the table overleaf. If unsure, take measurements of the clear vessel lumen and the residual diameter within the stenosis to aid assessment of the degree of stenosis (diameter reduction) using NASCET criteria.

NASCET = B-A x 100%

B

Report a 65% stenosis as per local protocol i.e.  65% if PSV 180.

In the presence of a significant stenosis with severe contralateral disease or occlusion, the report should explain that any elevated velocities present may be due to this side acting as a collateral pathway which may lead to elevated disease predictions.

| Diameter Reduction | # PSV | # EDV | PSVICA/ PSVCCA | St Mary’s ratio PSVICA/ EDVCCA | Oxford General Comments |
| --- | --- | --- | --- | --- | --- |
| 0-29 | 100 | 40 | 3.2 |  |  |
| 30-49 | 110-130 | 40 | 3.2 | <8 |  |
| 50-59 | 130 | 40 | 3.2 | 8-10 |  |
| 60-69 | 130 | 40-110 | 3.2-4.0 | 11-13 | 65% if PSV 180 |
| 70-79 | 230 | 110-140 | 4.0 | 14-21 | 70% if PSV 230, EDV may vary |
| 80-89 | 230 | 140 | 4.0 | 22-29 | 80% if PSV 300, EDV generally 85% if PSV >400 |
| >90 | >400 | >140 | >5 | >30 | 90% if PSV >500 |
| 96-99 | “STRING FLOW” |  |  |  |  |
| 100 | NO FLOW |  |  |  |  |

Total Occlusion and Sub-occlusion

Total occlusion may be reported when no flow is observed on colour flow image and no Doppler waveform can be detected. The CCA may demonstrate no flow during diastole.

If the vessel is patent but with very poor trickle flow, a sub-occlusion should be reported.

In the case of a total occlusion of the CCA the direction of flow in ECA-ICA should be noted. (It may be retrograde in the ECA).  Settings should be adjusted accordingly to assess low flow.

Plaque Morphology

Describe plaque morphology using Gray-Weale classifications3 but using the following wording:

## Smooth

- Irregular

- echolucent

- echogenic

- Mixed

- Ulcerated

Tortuous Carotid Artery

Note if vessels are tortuous or kinked. Where there is a ‘hairpin loop’ (180º turn on itself), this should also be reported.  If these significantly impact on flow, this should be reported.

Additional Information

Note incidental findings, e.g. enlarged thyroid, splayed carotid arteries. If a carotid body tumour or aneurysm is detected note the size, position and extent.

Scan Confidence

# Report confidence of scan results:

4-5/5 		clear scan

3/5 		difficult scan - may require repeat scan or alternative imaging

1-2/5 		very difficult scan, unable to obtain views

State in the report if the scan was suboptimal

Additional trainee requirements

In addition to the above, trainees will be expected to practice and demonstrate the following steps

- Start the scan with a preliminary mapping of the vessels and any disease in B-mode and colour Doppler by,

- Imaging the CCA, bifurcation, ICA and ECA in B-mode both longitudinally and in transverse.

- Imaging the CCA, bifurcation, ICA and ECA with colour Doppler both longitudinally and in transverse.

- Call for assistance if the preliminary assessment suggests a >50% stenosis.

- Image the subclavian arteries and assess for any proximal disease using colour Doppler and spectral Doppler.

# Patient Management

Normal

If normal scan or <50% ICA stenosis – no additional action required.

If very soft plaque or mobile thrombus is seen – bleep/telephone the referring clinician to notify them of the findings. If the referrer is uncontactable, then contact the on-call vascular registrar.

Abnormal (symptomatic >50% stenosis)

Bleep/telephone the referring clinician to notify them of the findings. If the referrer is uncontactable, then contact the on-call vascular registrar.

Significant disease (more than 60% stenosis)

If asymptomatic, no additional action required.

If the patient has been symptomatic recently (within last thirty days)

- Ask the patient to wait in the VSU waiting room while you discuss the result with the on-call vascular team who should see the patient if possible.

- Bleep/telephone the referring clinician to notify them of the findings and that you have contacted the vascular surgical team as per protocol.

- Document on the report the outcome of any discussion. (this can be hand written)

Pre-Carotid Endarterectomy

Patients may require a repeat scan within 24 hours prior to carotid endarterectomy to check patency of internal carotid artery, assess extent of stenosis, calibre of the distal vessel and mark the bifurcation.  High or low bifurcations are noted.

Document on the report the length of plaque and the diameter of the mid ICA.

References

1 Oates, CP, Naylor AR, Hartshorne, T et al.  Joint Recommendations for Reporting Carotid Ultrasound Investigations in the United Kingdom

2 Sidhu PS, Allan PL. Ultrasound assessment of internal carotid artery stenosis. Clin Radiol. 1997;52:656.

3 Gray-Weale AC, Graham JC, Burnett JR, Bryne K, Lusby RJ. Carotid artery atheroma: comparison of preoperative B-mode ultrasound appearance with carotid endarterectomy specimen pathology. J Cardiovasc Surg. 1988;2:676–681.